Production Planning Detailed Scheduling

Summary

- Application Flow diagram:

- Create → Production order is generated from

- demand/scheduler inputs.

- Plan → Batch, resources, and production sequence are assigned.

- Execute → Manufacturing process starts and shop-floor data is captured.

- Confirm → Production completion and quantities are confirmed in the system.

- Quality → Samples, results, and usage decisions are recorded.

- Store → Finished goods are moved to the final storage location.

- Update Systems → SAP, CPI, L2, and other interfaces are updated with final transaction data.

- Create → Plan → Execute → Confirm → Quality → Store → Update Systems

Apriso SAP Integration:

- Apriso and SAP are connected through CPI middleware.

- CPI works like a bridge that sends data between both systems.

- SAP → Apriso (Inbound): SAP sends order-related data like batch allocation, inventory, sales orders, and quality updates.

- Apriso → SAP (Outbound): Apriso sends shop-floor updates like order confirmation, order reversal, UD posting, and reversals.

- This ensures real-time data sync between ERP and manufacturing operations.

- It helps both systems stay updated with production, inventory, and quality information.

Flow in One Line

- SAP sends order data → CPI transfers → Apriso executes → Apriso sends updates back → SAP gets final status

- This is the simple communication flow between ERP and shop floor systems.

Apriso L2 Integration:

- L2 systems (SMS, HSM, CRM) generate production line data.

- This data is first stored in the Production DB.

- Then it moves to the Staging DB for processing and formatting.

- After validation, the data is sent to MES Apriso.

- Apriso uses this data for manufacturing execution and shop-floor monitoring.

- Different lines/modules like CCS, HSM, Combo Line, HAPL, and CAPL send line-specific data.

- Some data is stored in separate columns, while some is stored in XML format.

- This integration ensures real-time production data flow from L2 systems to MES.

- Flow in One Line

- L2 Systems → Production DB → Staging DB → MES Apriso → Shop Floor Monitoring & Execution

- This is basically the real-time machine/line data integration flow into Apriso.

- Apriso-Quintiq:

- Quintiq sends planning/scheduling data to Apriso through a message bus.

- The data is shared in JSON list format.

- Apriso receives it through Active MQ Service Manager.

- Then business logic is executed, and the data is shown on Apriso execution screens.

- In reverse flow, Apriso sends execution updates back to Quintiq.

- Data is again converted into JSON format.

- It passes through endpoint → Active MQ → message bus.

- Quintiq receives the updated execution status for planning synchronization.

- Flow in One Line

- Quintiq Planning → Message Bus → Apriso Execution → JSON Update → Quintiq Sync

- Apriso-Quintiq Integration Flow:

- Sales order starts in SAP with customer order details.

- Order details are sent through CPI and Staging DB.

- DM creates the flexi order and converts it into a firm order.

- Firm orders are shared with OC and schedulers.

- Schedulers generate the sequenced firm order.

- The sequenced order is stored in Staging DB.

- Apriso receives the sequenced firm order for production execution.

- During execution, batch feedback, start, and completion status are sent back.

- If needed, manual batch allocation can be done for new firm orders.

- After UD (usage decision), the final production status is updated.

- Flow in One Line

- SAP Sales Order → CPI/Staging → DM Firm Order → Scheduler Sequence → Apriso Execution → Feedback Update